ELEC3042 Minor Project Report

Name: Tyler Johnson

MQID: 46978518

# Overview

In designing the stopwatch, I decided to handle the inputs in the ISR as it was important to the operation of the stopwatch that the inputs be precise. The main state machine handles switching between the three features of the stopwatch, paused, counting, and initial. The main loop is responsible for updating the digits[] array, which is sent to the 7 Segment Display by the sendData() function.

Figure 1 shows the system block diagram and the signals that flow between them.

Figure 1: System Block Diagram

# System Resources

To keep time in the system I used timer0 in CTC mode to keep time. Timer0 generates an interrupt every millisecond, which increments two variables, clock_count and debounce timer. This saved me having to use two timers to keep track of time for debouncing and for the clock. Changes on the input pins for S1-A1 and S2-A2 (PC1, PC2) generate an interrupt that simultaneously handles debouncing and setting flags that are read by the state machine (button_flags).

# PCINT1 Interrupt

PCINT1 is responsible for handling the inputs and interrupting the system to ensure they are read as quickly as possible. The buttons are debounced within the ISR to ensure inputs are only read once. A separate variable, debounce timer, is used to keep time as it makes it much easier to manipulate two separate variables instead of managing the same one.

# Main State Machine Diagram

Figure 2: State Machine Diagram

## Truth Table

Table 1: State Machine truth table

## State Machine

The stopwatch is required to have three different functions. An initial state (stateInitial) where the count is zero and the timer isn’t incrementing. A counting state (stateCount) which counts upwards from the time initially displayed when S2-A2 is pressed. And a paused state (statePaused) which pauses the timer and turns on LED4. It is also required to resume the count from wherever it was paused. The code for the State Machine is implemented in the main loop.

# sendData()

sendData() sends the given the segments from segmentMap() and the digits from digits[], sends the requisite signals to the 74HC595 8-bit shift register and latches them to the display.

The code above follows the general signal graph detailed below.

New data is sent from PB0 to the shift register on the rising edge of PD7. Once all the data is sent PD4 is set to high which latches all the values of

# Full Code

/*

* File: MinorProject.c

* Author: Tyler Johnson

*

* Created on 19th March 2022, 4:55 PM

*/

//==============Header files==============

#include <xc.h>

#include <avr/io.h>

#include <avr/interrupt.h>

#define A1 0b00000010                                   //Define variable for A1 pin

#define A2 0b00000100                                   //Define variable for A2 pin

//==== Global Variables and Functions ====

char digits[4];                                         //char array of length 4 - in initial state

volatile uint32_t clock_count = 0;                      //millisecond timer, this is updated every millisecond using a CTC timer interrupt

volatile uint32_t display_time = 0;                     //Time being displayed

volatile uint32_t saved_time = 0;                       //Time saved (during pause state)

volatile uint32_t A1_time = 0;                          //Time variable for storing time since A1 is pressed first

volatile uint32_t A2_time = 0;                          //Time variable for storing time since A1 is pressed first

volatile uint8_t old_button = 0b00001110;               //Old Button State

volatile uint8_t new_button = 0b00001110;               //New Button State

volatile uint32_t debounce_timer = 0;                   //another millisecond timer running off the same timer interrupt to keep track of time for debouncing

volatile uint8_t button_flags = 0;                      //Variable for setting button flags

enum STATE {stateInitial, stateCount, statePaused};     //Define each state for the state machine

enum STATE cur_state;                                   //Define a variable to keep track of the current state

enum STATE next_state;                                  //Define a variable to keep track of the state being transitioned into

//Segment byte maps for numbers 0 to 9

const uint8_t SEGMENT_MAP[] = {

0xC0, 0xF9, 0xA4, 0xB0, 0x99, 0x92, 0x82, 0xF8, 0X80, 0X90,

// Continuing on for A (10) to F (15)

0x88, 0x83, 0xC6, 0xA1, 0x86, 0x8E,

// Then blank (16), dash (17)

0xFF, 0x40

};

//Returns the corresponding segmap value

uint8_t segmentMap(uint8_t value) {

return SEGMENT_MAP[value];

}

//Interrupt for S1-A1 and S2-A2

ISR(PCINT1_vect) {

new_button = PINC;                                  //Current state of PINC

uint8_t changed_bits = old_button^new_button;       //Calculate which bits have changed

if (changed_bits & A1){                             //Check if button change has occurred on A1

if ((debounce_timer - A1_time)>3){              //If 30 ms has passed

A1_time = debounce_timer;                   //Set A1_time to equal the value of de-bounce (zero it)

if ((old_button & ~new_button) & A1){       //If there is still a change in A1

button_flags |= A1;                     //Set flag to high so it can be processed in loop

}

old_button = new_button;                    //Change the old button state to the new button state

}

}

if (changed_bits & A2){                             //Check if button change has occurred on A1

if ((debounce_timer - A2_time)>3){              //If 30 ms has passed

A2_time = debounce_timer;                   //Set A2_time to equal the value of de-bounce (zero it)

if ((old_button & ~new_button) & A2){       //If there is still a change in A2

button_flags |= A2;                     //Set flag to high so it can be processed in loop

}

old_button = new_button;                    //Change the old button state to the new button state

}

}

}

//Interrupt for timer

ISR(TIMER1_COMPA_vect) {

clock_count++;                                      //Increment clock count (every millisecond))

debounce_timer++;                                   //Increment debounce tracking variable

}

void timer1Setup (){

TCCR1A = 0b01000000;                                //CTC Mode

TCCR1B = 0b00001011;                                // /64 Prescaler

OCR1A = 2500;                                       //Number we are "counting" up to.

TCNT1 = 0;                                          //Number we are "counting" up from.

ICR1 = 0;                                           //Input Capture Register

TIFR1 = 0b00000000;                                 //Timer interrupt flag register

TIMSK1 = 0b00000010;                                //Timer interrupt mask

}

void setup() {

timer1Setup();   //Runs timerSetup

//DDR - 0 for input - 1 for output

//PORT - 0 for no pullup - 1 for pullup

DDRB = 0b00111101;          //Direction Register B - LEDs

PORTB = 0b00111100;         //PORTB Internal Pullups

DDRC = 0b00000000;          //Direction Register C - Attached to buttons and the turnpot

PORTC = 0b00001110;         //PORTC Internal Pullups

DDRD = 0b10010000;          //Direction Register D - For clocks

PORTD = 0b00000000;         //PORTD Internal Pullups

PCMSK1 = 0b00001110;        //Pin Change Mask Register 0

PCICR = 0b00000010;         //Pin Change Interrupt Control Register

sei();                      //Enable global interrupts

}

//Sending data to the 7 segment display using segments and digit array as input

void sendData(uint8_t segments, uint8_t digits) {

PORTD |= _BV(7);                            //Set PD7 High

for (int i = 0; i < 8; i++){                //Loop 8 times

PORTD &= ~_BV(7);                       //Set PD7 Low

if(segments & (0b10000000>>i))      //Compare segments to a mask that shifts based on the value of i

{

PORTB |= _BV(0);                //Set PB0 High

}

else

{

PORTB &= ~_BV(0);               //Set PB0 Low

}

PORTD |= _BV(7);                        //Set PD7 High

}

for(int j = 0; j < 8; j++){                 //Loop 8 times

PORTD &= ~_BV(7);                       //Set PD7 Low

if(digits & (0b10000000>>j))

{

PORTB |= _BV(0);                //Set PB0 High

}

else

{

PORTB &= ~_BV(0);                //Set PB0 Low

}

PORTD = 0b10000000;

}

PORTD = 0b10010000;

}

//Sends data from digits to the sendData function - which in turn

//displays on the 7 segment

void showDigits() {

sendData(segmentMap(digits[0]) & 0b01111111, (1<<0));   //Sends data and enables dp

sendData(segmentMap(digits[1]), (1<<1));

sendData(segmentMap(digits[2]) & 0b01111111, (1<<2));   //Sends data and enables dp

sendData(segmentMap(digits[3]), (1<<3));

sendData(segmentMap(16), 0);                            // blank the display

}

//===Main Loop==//

int main(void) {

setup();

cur_state = stateInitial;                       //Set state to Initial off the bat

while(1) {                                      //Main Loop

showDigits();                               //Displays digits (0.00.0)

switch(cur_state){                          //Switch statement

case stateInitial:                      //Initial State - timer is at 0 and count doesn't increment

PORTB |= 0b11111111;                //Turn off all LEDs

clock_count = 0;                    //Reset the time

display_time = 0;                   //Change the saved time to the current display time (0)

if(button_flags & A1){                      //If A1 is pressed

button_flags = button_flags & ~A1;      //Reset the flag

next_state = stateInitial;              //Set the next state to initial

}

else{

if(button_flags & A2){                  //If A2 is pressed

next_state = stateCount;            //Set the next state transition to stateCount

button_flags = button_flags & ~A2;  //Reset the flag

}

}

break;

case stateCount:                                //Count State – Timer counting up from paused or initial

PORTB |= 0b11111111;                        //Turn off all LEDs

display_time = clock_count;                 //Assign clock count to display_time

if(button_flags & A1){                      //If A1 is pressed

button_flags = button_flags & ~A1;      //Reset the flag

next_state = stateInitial;              //Set the next state transition to stateInitial

}

else{

if(button_flags & A2){                  //If A2 is pressed

button_flags = button_flags & ~A2;  //Reset the flag

saved_time = display_time;          //Save the value of display_time

next_state = statePaused;           //Set the next state transition to statePaused

}

else{

next_state = stateCount;            //Return to stateCount

}

}

break;

case statePaused:                               //Pauses count and displays the same value, when start/stop is pressed it resumes

PORTB &= 0b11111011;                        //Turn on D4 LED

if (button_flags & A2){                     //If A2 is pressed

button_flags = button_flags & ~A2;      //Reset the flag

clock_count = saved_time;               //Assign saved_time to clock_count

next_state = stateCount;                //Set the next state transition to stateCount

}

else{

next_state = statePaused;               //Otherwise stay in statePaused

if (button_flags & A1){                 //If A1 is pressed

button_flags = button_flags & ~A1;  //Reset the flag

next_state = stateInitial;          //Set the next state transition to stateCount

clock_count = 0;                    //Reset clock_count to 0

}

else{

if ((button_flags & A2)==1){            //If A2 = button_flags

button_flags = button_flags & ~A2;  //Reset the flag

next_state = statePaused;           //Set the next state transition to statePaused

}

}

}

break;

}

cur_state = next_state;                             //set cur_state to next_state

//update display_time to change the time displayed on the 7 segment display

digits[0] = ((display_time/10)/600)%10;

digits[1] = ((display_time/100)%60)/10;

digits[2] = ((display_time/100)%60)%10;

digits[3] = (display_time/10)%10;

showDigits();

}

}